Beetle Bot Manual

Phase 1: Workspace & Environment Setup

Before touching the robot, you need to prepare your local development environment.

- Create Workspace: Open your terminal and create the directory structure:

  - $ mkdir -p ~/ros2_ws/src (Replace ros2_ws with your register number).

- Initialize & Build:

  - Navigate to the folder: $ cd ~/ros2_ws.

  - Source the ROS 2 Jazzy environment: $ source /opt/ros/jazzy/setup.bash.

  - Build the workspace: $ colcon build.

- Automate Sourcing: Add your workspace to your bash configuration so it loads every time you open a terminal:

  - $ echo "source ~/ros2_ws/install/setup.bash" >> ~/.bashrc.

Phase 2: Turtlesim Simulation (Testing)

Use this to ensure your ROS 2 installation is working correctly.

- Install Turtlesim:

  - $ sudo apt update.

  - $ sudo apt install ros-jazzy-turtlesim.

- Run Simulator:

  - In a new terminal, source the environment.

  - Launch the node: $ ros2 run turtlesim turtlesim_node.

- Teleoperation (Control):

  - Open another terminal and source it.

  - Run the keyboard controller: $ ros2 run turtlesim turtle_teleop_key.

  - Alternative: Use the general keyboard teleop with a remap: $ ros2 run teleop_twist_keyboard teleop_twist_keyboard --ros-args --remap cmd_vel:=my_cmd_vel.

- Topic Data view :

Ros2 topic list

Ros2 topic echo

- Record topic data:

ros2 bag record /cmd_vel

ros2 bag record -o my_cmd_vel_bag /cmd_vel

ros2 bag record /scan

ros2 bag record -o scan_data_bag /scan

ros2 bag record -a

ros2 topic echo /scan > scan_output.txt

Phase 3: BeetleBot Hardware Connection

Now, let’s bridge your PC to the actual robot hardware.

- Network Connection:

  - Turn on the bot and connect your PC to the Wi-Fi network: BEETLEBOT_5G.

  - Enter the password: 15619xxx.

- SSH into Robot:

  - Open a new terminal and type: $ ssh veerobot@192.168.0.bot_No (Example: $ ssh veerobot@192.168.0.129).

  - Use the password: veerobot.

- Set Domain ID (Critical Step):

  - On the Robot: Source the workspace and set your unique ID:

    - $ source ~/lyra_ws/install/setup.bash.

    - $ export ROS_DOMAIN_ID=129 (Use your specific robot number).

  - On your PC: Run the exact same two commands in every new terminal you open to ensure the PC can "see" the robot.

Phase 4: Launching BeetleBot Operations

- Start the Robot (Bringup):

  - In the SSH terminal (on the robot), run:

    - $ cd lyra_ws/.

    - $ ros2 launch lyra_bringup robot.launch.py mode:=slam camera:=true imu:=true.

- Verify Topics (On PC):

  - Open a new terminal on your PC and check the active data streams:

    - $ ros2 topic list.

    - To see specific data: $ ros2 topic echo /scan or $ ros2 topic echo /imu/data_raw.

+1

[!NOTE]

Technical Check: The manual mentions $ ros2 topic node. This is likely a typo for $ ros2 node list, which will show you all active nodes running on the network.

Phase 5: Visualization & Data Management

- Visual Tools:

  - To see the robot's sensors in 3D: $ ros2 run rviz2 rviz2.

  - To see the camera feed: $ rqt.

- Recording Data:

  - Record a specific topic: ros2 bag record -o scan_data_bag /scan.

  - Save a text log of the scan: ros2 topic echo /scan > scan_output.txt.

- Save Your Map:

  - Install the map server: $ sudo apt install ros-jazzy-nav2-map-server.

  - Save the result: ros2 run nav2_map_server map_saver_cli -f ~/my_map.

*******************************************************************************

copy files from your PC to a Raspberry Pi using SSH in the terminal, you can use the scp (secure copy) command.

scp <local_file_path> <username>@<raspberry_pi_ip>:<remote_path>

Example

If your Raspberry Pi username is pi, IP is 192.168.1.50, and you want to copy test.txt to the Pi home folder:

scp test.txt pi@192.168.1.50:/home/pi/

Copy a Folder

To copy a full folder, use -r (recursive):

scp -r myfolder pi@192.168.1.50:/home/pi/

Copy Only Files (all types) from a Path

If you want to copy all files in a folder but not the folder itself:

scp /path/to/folder/* pi@192.168.1.50:/home/pi/destination/

Example:

scp ~/documents/* pi@192.168.1.50:/home/pi/documents/

This copies all file types (.txt .py .jpg .pdf .cpp etc.).

*******************************************************************************